DMZ Zone Project

Network Security Architecture Documentation

Cisco Packet Tracer Lab

# 1. Project Overview

This document covers the full configuration of a multi-zone network security lab built in Cisco Packet Tracer. The design separates the Internal LAN, DMZ, Edge, and Internet zones to simulate a real-world enterprise network with controlled traffic flow between each segment.

## 1.1 Network Zones Summary

| Internal LAN | 192.168.1.0/24 — Internal hosts (PC0–PC3, INTERNAL-SERVER1) |
| --- | --- |
| DMZ | 172.16.1.0/24 — DMZ-SERVER1 (.1), DMZ-SERVER2 (.2) |
| Core-to-DMZ Link | 10.0.0.0/30 — CORE_SWITCH (.254) to DMZ Router (.2) |
| Edge Link | 2.2.2.0/28 — DMZ Router (.2) to ISR4331 EDGE (.1) |
| Internet Link | 1.1.1.0/30 — EDGE (.2) to INTERNET Router (.1) |
| Internet Simulated | 100.100.100.0/24 — Remote User (.4), Google (.2), Internet Server (.3) |
| Routing Protocol | OSPF Area 0 (Internal) + Static Routes (DMZ) + BGP (Edge/Internet) |

# 2. Topology & Device Roles

## 2.1 Internal LAN

- 3 Access switches (ACCESS-SW1, ACCESS-SW2, ACCESS-SW3) connecting PC0, PC1, PC2, PC3, and INTERNAL-SERVER1

- All uplinks feed into CORE_SWITCH (3650-24PS) via Gig interfaces

- CORE_SWITCH is the OSPF-speaking boundary between the LAN and the DMZ router

## 2.2 DMZ Zone

- DMZ-SWITCH connects DMZ-SERVER1 (172.16.1.1) and DMZ-SERVER2 (172.16.1.2)

- DMZ Router handles routing between the DMZ, the CORE_SWITCH, and the EDGE router

- Static routes are used on the DMZ router instead of dynamic routing — intentional design decision for security

## 2.3 Edge & Internet

- ISR4331 EDGE (BGP AS200) connects the DMZ network outward to the Internet router (BGP AS100)

- Internet zone simulates external users: Remote User (.4), Google (.2), Internet Server (.3)

- Loopbacks 7.7.7.7, 8.8.8.8, 9.9.9.9 simulated on the Internet router

# 3. Routing Design

## 3.1 OSPF Configuration (Area 0)

OSPF is used to distribute routing information within the Internal LAN and the link toward the DMZ. Only the following networks are advertised into OSPF Area 0:

| Internal LAN | 192.168.1.0/24 — advertised from CORE_SWITCH |
| --- | --- |
| Core-to-DMZ Link | 10.0.0.0/30 — advertised on both CORE_SWITCH and DMZ Router |

OSPF Adjacency on the CORE_SWITCH

OSPF Adjacency on the DMZ_ROUTER

## 3.2 Static Routes on the DMZ Router

Static routes are configured on the DMZ Router to control how traffic flows back toward the internal LAN. Dynamic redistribution is intentionally avoided here — the DMZ should not advertise routing information outward. This keeps the internal topology hidden and adds a layer of security.

On the DMZ Router, a default static route (0.0.0.0/0) points toward the EDGE router. This ensures any unknown destination (internet-bound traffic) is forwarded to the edge.

## 3.3 BGP (Edge to Internet)

- ISR4331 EDGE operates in BGP AS200

- Internet Router operates in BGP AS100

- BGP peers over the 1.1.1.0/30 link

- Internet loopbacks (7.7.7.7, 8.8.8.8, 9.9.9.9) are reachable via BGP advertisement

# 4. NAT & Port Forwarding

## 4.1 Overview

NAT (Network Address Translation) is configured on the DMZ Router to allow external users to reach internal DMZ servers using the public IP 2.2.2.2. Port forwarding (static NAT with TCP port mapping) is used so specific ports are redirected to the correct server.

## 4.2 Port Forwarding Table

| External IP | External Port | Destination Server | Internal Port |
| --- | --- | --- | --- |
| 2.2.2.2 | 80 | DMZ-SERVER1 (172.16.1.1) | 80 |
| 2.2.2.2 | 443 | DMZ-SERVER1 (172.16.1.1) | 443 |
| 2.2.2.2 | 8080 | DMZ-SERVER2 (172.16.1.2) | 80 |
| 2.2.2.2 | 4443 | DMZ-SERVER2 (172.16.1.2) | 443 |

## 4.3 NAT Commands

The same external port cannot be used twice in NAT. Since DMZ-SERVER1 already occupies ports 80 and 443, DMZ-SERVER2 uses alternate external ports (8080 and 4443):

ip nat inside source static tcp 172.16.1.1 80 2.2.2.2 80

ip nat inside source static tcp 172.16.1.1 443 2.2.2.2 443

ip nat inside source static tcp 172.16.1.2 80 2.2.2.2 8080

ip nat inside source static tcp 172.16.1.2 443 2.2.2.2 4443

Interfaces must also be marked:

interface [DMZ-facing interface gigabitethernet 0/0/0]  →  ip nat inside

interface [EDGE-facing interface gigabitethernet 0/0/0] →  ip nat outside

# 5. Access Control Lists (ACLs)

## 5.1 DMZ Access Control — External (Remote User)

Only the Remote User (100.100.100.4) is permitted to reach DMZ servers via web ports. All other traffic is denied. The following ACL is applied inbound on the DMZ Router's external interface:

permit tcp any host 2.2.2.2 eq www

permit tcp any host 2.2.2.2 eq 443

permit tcp host 100.100.100.4 host 2.2.2.2 eq 8080

permit tcp host 100.100.100.4 host 2.2.2.2 eq 4443

This allows any external host to reach DMZ-SERVER1 on standard web ports (80/443), while restricting ports 8080 and 4443 (DMZ-SERVER2) specifically to the Remote User.

## 5.2 Internal LAN Access Control

ACLs are also applied to control what the Internal LAN can reach in the DMZ:

- Internal hosts CAN reach DMZ-SERVER1

- Internal hosts CANNOT reach DMZ-SERVER2

OSPF Adjacency Preservation

When configuring the ACL on the internal-facing interface, OSPF hello packets were originally being denied along with other traffic. This broke the OSPF neighbor relationship between CORE_SWITCH and the DMZ Router.

The fix is to explicitly permit OSPF traffic in the ACL before the deny statements. OSPF uses IP protocol 89:

permit ospf any any

This line must come before any deny statements that would otherwise block OSPF hellos and DBD packets, which would tear down the neighbor adjacency.

PRO TIP: Always sequence the ACL to permit routing protocol traffic (OSPF/EIGRP/BGP) before applying broad deny rules on inter-zone interfaces. Losing OSPF adjacency mid-config is a common gotcha.

# 6. Verification Commands

| Command | Purpose |
| --- | --- |
| show ip ospf neighbor | Verify OSPF adjacency between CORE_SWITCH and DMZ Router |
| show ip route | Confirm routing table entries including OSPF and static routes |
| show ip nat translations | Verify active NAT/PAT entries and port forwarding |
| show access-lists | Review ACL hit counts to confirm traffic is matching rules |
| ping 172.16.1.1 source 192.168.1.x | Test LAN-to-DMZ-SERVER1 reachability |
| ping 172.16.1.2 source 192.168.1.x | Should FAIL — confirms DMZ-SERVER2 is blocked from LAN |
| show ip bgp summary | Verify BGP peer state between EDGE and INTERNET routers |

# 7. Security Design Notes

- Static routes on the DMZ Router are used instead of redistributing OSPF outward. This ensures the internal network topology (192.168.1.0/24) is never advertised beyond the DMZ boundary.

- The DMZ is a controlled buffer zone — servers here are publicly reachable but completely isolated from the internal LAN by ACL policy.

- Port-forwarding via NAT provides an additional layer of obfuscation. External users interact only with 2.2.2.2 and never see the actual internal DMZ server IPs.

- OSPF is confined to Area 0 between the Internal LAN and the DMZ-facing link. BGP handles edge-to-internet routing independently.

- ACLs are sequenced carefully to avoid dropping control-plane traffic (OSPF hellos) while still enforcing data-plane policy.

DMZ Zone Project — Cisco Packet Tracer Lab | Franck Kashindi